RESETTING TRIPS IN DR and RICH

- Open the  Detector-Based Adjustment Frame GUI

- If there are channels listed in the lower half of the window, press ‘Clear All’ for the system they belong to.

- Select the tripped channels.

- Right click on BEN column and change value to 0 to disable bulk.

- Hit ‘Refresh’ a few times to make sure the BEN column goes to 0

- Continue to Hit ‘Refresh’ until BMV is below 100 Volts

- Right click on CEN column and change value to 1 to reset the trip.

- Hit refresh several times and make sure trip is reset.

- Right click on BEN column and change value to 1 to ramp up the HV.

- Any Questions? PLEASE CALL THE HV EXPERT!